import numpy as np

import pandas as pd

def pade_approx(theta, order=2):

"""Xấp xỉ khâu trễ e^(-theta*s) theo Padé bậc 1 hoặc bậc 2"""

if theta <= 1e-4:

return [1.0], [1.0]

if order == 1:

return [-theta / 2.0, 1.0], [theta / 2.0, 1.0]

elif order == 2:

return [theta**2 / 12.0, -theta / 2.0, 1.0], [theta**2 / 12.0, theta / 2.0, 1.0]

def calc_step_metrics(t, y, y_final=None):

"""Tính các chỉ tiêu chất lượng quá độ"""

if y_final is None:

y_final = y[-1]

y_max = np.max(y)

peak_time = t[np.argmax(y)]

overshoot = max(0.0, (y_max - y_final) / abs(y_final) * 100.0) if abs(y_final) > 1e-5 else 0.0

tol = 0.02 * abs(y_final) if abs(y_final) > 1e-5 else 0.02

settled_idx = np.where(np.abs(y - y_final) > tol)[0]

ts = t[settled_idx[-1]] if len(settled_idx) > 0 and settled_idx[-1] < len(t) - 1 else t[-1]

idx10 = np.where(y >= 0.1 * y_final)[0]

idx90 = np.where(y >= 0.9 * y_final)[0]

tr = t[idx90[0]] - t[idx10[0]] if len(idx10) > 0 and len(idx90) > 0 else 0.0

ess = abs(1.0 - y_final)

return {"OS": overshoot, "Tp": peak_time, "Ts": ts, "Tr": tr, "Ess": ess, "Yfinal": y_final}

def compute_routh_table(coeffs):

"""Tính bảng tiêu chuẩn đại số Routh-Hurwitz"""

coeffs = [float(c) for c in coeffs]

n = len(coeffs)

if n == 0:

return [], True, 0

rows = n

cols = (n + 1) // 2

table = np.zeros((rows, cols))

for i, c in enumerate(coeffs):

table[i % 2, i // 2] = c

for r in range(2, rows):

for c in range(cols - 1):

a = table[r-1, 0]

if abs(a) < 1e-9:

a = 1e-5

b = table[r-2, 0]

c1 = table[r-2, c+1]

c2 = table[r-1, c+1]

table[r, c] = (a * c1 - b * c2) / a

first_col = table[:, 0]

sign_changes = 0

prev_s = np.sign(first_col[0])

for val in first_col[1:]:

s = np.sign(val)

if s != 0 and s != prev_s:

sign_changes += 1

prev_s = s

is_stable = (sign_changes == 0) and all(c > 0 for c in coeffs)

return table, is_stable, sign_changes